BRA CX3 Honda powered “cycle car”.

# BRA Merlin CX3

01273 843749 07976 312058

art@spitfireart.com

www.bra-cars.com

www.spitfireart.com

www.veetwintrikes.com

Specifications for professionally built car.

New! Caterham Seven front suspension, steering rack & column.

Engine/transmission:

Honda CX 500 water cooled V-Twin 500cc engine with integral manual sequential 5- speed gearbox.

Extended CX 500 drive shaft with additional bearings. BRA gear linkage complete with rose jointed ends. Engines are professionally re-built by Cxpertise, a Honda engine specialist.

A reversing gearbox will be available as an optional extra. This element is still under development. Both mechanical and electrical types are being evaluated.

Chassis:

Mig welded jig built space frame predominantly in 25mm square 16swg tubing. Chassis prepared by sand blasting and powder coating in choice of colour. Front and rear protrusions to be over painted in choice of solid body work colour.

Additional items to be included with chassis:

BRA pedals and pedal box. Location of pedal box is adjustable and pre-drilled.

Ford Escort hand brake lever.

BRA handbrake linkage.

BRA cycle wing stays black powder coated

Transmission tunnel covers.

BRA stainless steel fuel tank.

Body work:

16 swg aluminium alloy body panels and bulkheads. 16 swg one-piece floor.

GRP nose cone (new rounded design), scuttle, upper rear body cover (hinged to open) and cycle wings.

Aluminium  multi-louvred opening bonnet with stainless steel hinge.

All body work to be prepared, etch primed and finished in choice of solid colour in two pack paint.

Alternative overall finish:

All GRP mouldings finished in solid colour two pack paint. Aluminium panels  in bright polished finish.

Additional items included with bodywork:

BRA stainless steel exhaust system complete with silencers.

Chrome bonnet catches and rear cover lock.

Stainless steel body panel fixings.

BRA seat set in black vinyl.

BRA vinyl tonneau cover and tenax fittings.

Full V- screen.

BRA mirror set

Large capacity radiator (Reliant) complete with electric fan and thermal switching unit.

BRA speedometer drive unit and cable.

BRA tachometer cable.

Lighting:

Chrome 7” front head lamp units complete with side lights.

Chrome motor cycle indicators fitted to top of cycle wings.

Chrome Model A style rear lights with stop tail, number plate illumination and indicators.

Steering:

Ford Escort Mk 2 steering rack with BRA column. Now changing to Caterham Seven rack and adjustable steering column

Suspension and Brakes:

Front: New: Caterham Seven front suspension. BRA braided stainless steel brake hoses. BRA fabricated double unequal length wishbones with adjustable coil over spring damper units.

Rear: Honda CX 500 swing arm and bevel box (fully reconditioned) with twin coil over spring damper units. Hydraulic rear brake.

Wheels:

Front: 4J x 15” wire silver powder coated spoked rims with adapters and chrome spinners.

N.B. Chrome wire wheels compliment a polished aluminium body-work finish. Additional prices relate.

Rear: Honda 18” Comstar wheel converted to accept 4J x 15” tyre.

Spare: Same as front wheel combination.

Cockpit finish and trim:

Dashboard constructed from 12mm plywood, routed with curved edge and mahogany veneer finish. Full instrumentation and switches. Fitted carpets complete with map pockets. Moto-Lita three spoke steering wheel.

Seat belts will be fitted according to current legislation. The car will be SVA procedure compliant.

# Total cost: £9750

## The Quaife reversing gearbox option will add approximately £895 to the above price.

### Individually sliding bucket seats are also an optional extra and afford additional leg length with the overall effect that the driver sits lower within the cockpit. See the list of optional extras.